AD30NA4L（R100）

Feature

- Onboard Intel Alder Lake-N serial processor

- 1*DDR5 SO-DIMM，Max 16GB

- 4*2.5G network ports,choose POE

- TPM2.0，1*M.2 2280 SSD，MiniPCIe(WIFI/BT/4G/mSATA)

- HDMI2.0+DP,1*Type-C（USB+DP）

- DC 12-19V；Size：120x120mm

Specification

| Processor | - Onboard Intel Alder Lake-N serial processor,TDP 15W |
| --- | --- |
|  | - EFI BIOS |
| Ram | - 1*DDR5 SO-DIMM，Max 16GB |
| Rom | - 1*M.2 M-Key 2280(NVMe PCIe 4.0 x2/SATA3.0 protocol) |
|  | - 1*SATA3.0 slot，2Pin 5V power |
| Display port | - 1*HDMI2.0,support 4096x2160@60Hz |
|  | - 1*DP,support 7680x4320@60Hz |
|  | - Type-C,support DP 7680x4320@60Hz and USB3.2 Gen2 |
| I/O interface | - Power button,reset button,2*USB3.0,2*USB2.0,TF card slot |
|  | - DC JACK,HDMI+DP,Type-C |
|  | - 4*2.5G LAN（3*Intel 226-V,1*RTL8156B，choose POE or not）,RJ45 console |
| Extended Interface/Function | - External TPM2.0 is optional, but not by default. The default is CPU built-in TPM2.0 |
|  | - 1*MiniPCIe（PCIe+USB2.0+SATA3.0 protocol，WIFI/BT/mSATA module）,Micro SIM slot |
|  | - 5*RS232 header，pitch 2.0mm；4*GPIO |
|  | - 1*4Pin PWM CPU FAN |
| Power | - DC 12-19V，over 36W |
| working environment | Working temperature: -20℃ ~ +60℃; Working humidity: 5% ~ 90% |
|  | Storage temperature: -40℃ ~ +85℃; storage humidity: 5% ~ 90% |
| operating system support | - Windows10，Windows11，Linux |
| Size | - 120x120 mm |
| net weight | - About 120g without radiator, about 220g with radiator |

AD30NA4L（R100）

Size                                              Unit：mm

Order Information

| Model | Optional | Optional Description |
| --- | --- | --- |
| AD30NA4L | - Onboard Intel Alder Lake-N serial processor | N95/N100/N300 |
|  | - POE | POE optional |
|  | TPM2.0 | External TPM2.0 optional |