GN41NA（R120）

Feature

◆ Onboard Intel Gemini Lake/Gemini Lake-R processor

◆ support TPM2.0(optional)

◆ 2*I225 2.5G Lan card

◆ 1*VGA，1*HDMI 2.0

◆ 6*COM port； Onboard eMMC Storage Chip optional

◆ DC 12-19V

Specification

| CPU | Integrated Intel Gemini Lake/Gemini Lake-R series CPU |
| --- | --- |
|  | EFI BIOS |
| Memory | 1*DDR4 SO-DIMM，max support 8GB |
| Display | 1*VGA，Support Resolution 1920x1200 |
|  | 1*HDMI2.0，Support Resolution 4096x2304@60HZ，Support 4K display |
|  | HDMI+VGA Synchronous or asynchronous display |
| Storage | 1*SATA3 |
|  | 1*mSATA |
|  | Onboard eMMC,32GB/64GB/128GB optional |
| Lan port | 2*Intel i225V 2.5G lan card |
| I/O port | 1*VGA，1*HDMI2.0 |
|  | 2*RJ45 |
|  | 2*USB3.0 |
|  | 2*DB9 RS232 |
|  | 1*DC JACK |
| expansion interface/function | 0-255 seconds, watchdog routine provided |
|  | Aluminum fanless radiator (mainboard reserved fan interface can support automatic and manual fan speed adjustment) |
|  | 4*USB2.0 pin header，2x5Pin，space 2.0mm |
|  | 6*COM port（4*RS232，2*RS232/RS485 optional），RS485 Supports automatic flow direction control |
|  | 8 bit GPIO，，2x5Pin，Space 2.0mm |
|  | 1个MiniPCIe socket，Support PCIe and USB2.0 device |
| Power supply | DC12-19V |
| Environment | Working temperature：-20℃ ~ +60℃；Working humidity:0% ~ 90% (non-condensing) |
|  | Storage temperature：-40℃ ~ +85℃；Storage humidity:0% ~ 90% (non-condensing) |
| OS | Windows10，Linux |
| Dimension | 120x120mm |
| Weight | With heatsink 260g，without heatsink 130g |

GN41NA（R120）

Size                                                Unit：mm

Ordering Information

| Module | Optional | Optional description |
| --- | --- | --- |
| GN41NA | Integrated Intel Gemini Lake/Gemini Lake-Rseries CPU | J4125/J5005/N4100/N5000 |
|  | TPM2.0 | optional |
|  | COM port optional | 2com/6com |
|  | Onboard storage eMMC | 32GB/64GB/128GB optional |