KU250Z3AW（R120）

Feature:

◆ Onboard Intel 6/ 7th Kaby lake-U series processor

◆ Onboard DDR4 Ram and one Ram socket

◆ VGA/HDMI/LVDS/EDP display

◆ Single and dual network optional, support AMT (requires CPU with Vpro platform function and Intel I219-LM network card)

◆ DC9-36V

Specification

| CPU | Onboard Intel Core 7th Kaby lake-U series SOC |
| --- | --- |
|  | EFI BIOS |
| Memory | Onboard 4GB/8GB DDR4 2133MHz；can add a Ram socket,max support 16GB |
| Display | Dual channel 24bit LVDS，1920x1200；EDP，4096x2304@60Hz（choose one of two） |
|  | VGA，2048x1536 |
|  | HDMI1.4，4096x2304@24Hz，4K display |
|  | Dual display, triple display, synchronous or asynchronous display |
| Storage | 1*SATA3 |
|  | Onboard eMMC，32GB/64GB/128GB optional |
|  | 1*MiniPCIe mSATA，Support SANDISK protocol |
| I/O ports | 1*VGA，1*HDMI |
|  | 1*RJ45，2*USB3.0 |
|  | 1*MIC-IN/LINE-OUT Audio |
|  | 1*DC Jack |
| Expansion | 1*Intel i225-V Lan card（When the I/O interface is 2 USB, the network card is 1 Intel i225-V network card and 1 Intel I219-LM Gigabit network card） |
|  | 0-255 seconds watchdog, provide watchdog routine |
|  | Aluminum fan radiator (supports automatic and manual fan speed adjustment) |
|  | 2*USB3.0，6*USB2.0，4 of which are USB2.0 pin headers |
|  | 5*RS-232，1*RS422/485 optional，RS485 Supports automatic flow direction control |
|  | 1*PS/2 pin connector |
|  | Support touch screen 4wire 5wire 8wire（optional） |
|  | 8 bit GPIO |
|  | 2*MiniPCIe slots, support PCIe and USB devices (MiniPCIe2 only supports PCIe protocol devices when touch screen is available) |
| Power supply | DC9-36V |
| Environment | Working temperature：-20℃ ~ +60℃；Working humidity:0% ~ 90% (non-condensing) |
|  | Storage temperature：-40℃ ~ +85℃；Storage humidity:0% ~ 90% (non-condensing) |
| OS | Windows10，Linux |
| Dimension | 155x117mm |
| Weight | With heatsink 280g，without heatsink 160g |

KU250Z3AW（R120）

Size                                               Unit：mm

Ordering information

| Module | Optional | Optional description |
| --- | --- | --- |
| Onbaord Intel Core 6/7th Kaby lake-U series SOC | CPU | Celeron 3865/ I3-6100U/I3-7100U I5-6200U/I5-7200U I7-6500U/I7-7500U |
|  | Onboard Ram | 4GB/8GB optional |
|  | Onboard Storage | 32GB/64GB/128GB optional |
|  | Lan card | 1lan/2lan |